Statement of Basis - Narrative

NSR Permit

Type of Permit Action:  	Regular-New

Facility:	MV 400 TPH Astec HMA Plant

Company:	Mesa Verde Enterprises

Permit No(s).:	9079M1

Tempo/IDEA ID No.:	39845 - PRN20240001

Permit Writer:	Joseph Mashburn

Fee Tracking (not required for Title V)

| Tracking | NSR tracking entries completed: [X] Yes [] No |
| --- | --- |
|  | NSR tracking page attached to front cover of permit folder: [X] Yes [] No |
|  | Paid Invoice Attached: [x] Yes [] No |
|  | Balance Due Invoice Attached: [] Yes [x] No |
|  | Invoice Comments: Paid in Full 3/04/2024 |

| Permit Review | Date to Enforcement: N/A | Date of Enforcement Reply: N/A |
| --- | --- | --- |
|  | Date to Applicant: 3/13/2024; 5/15/2024 | Date of Applicant Reply: 3/20/2024; 5/16/2024 |
|  | Date to EPA: N/A | Date of EPA Reply: N/A |
|  | Date to Supervisor: 3/8/2024, 5/15/2024 |  |

- Plant Process Description:

The function of the facility is to produce hot mix asphalt.  The 400 tph hot mix asphalt plant will  include: aggregate/RAP storage piles, a 5-bin cold aggregate feeder, scalping screen, pug mill, mineral filler silo with screw conveyor and baghouse, drum dryer/mixer with baghouse, incline conveyor, asphalt silo, asphalt heater, six (6) transfer conveyors, Evotherm storage tank, and two (2) asphalt cement storage tanks.

The HMA plant will be co-located with GCP-2-3269 aggregate crushing and screening plant operated by Toro Rock Products, LLC.  Toro Rock will be providing the aggregate used in the MV 400 TPH Astec HMA Plant asphalt mix.

- Description of this Modification:

This is a new facility.

- Source Determination:

1. The emission sources evaluated include: Hot Mix Asphalt Plant and co-located Toro Rock GCP2 Aggregate Crushing and Screening plant.

2. Single Source Analysis:

  - SIC Code: Do the facilities belong to the same industrial grouping (i.e., same two-digit SIC code grouping, or support activity)? No. The Aggregate Crushing and Screening plant has a different SIC Code.

  - Common Ownership or Control: Are the facilities under common ownership or control? No

  - Contiguous or Adjacent:  Are the facilities located on one or more contiguous or adjacent properties? Yes

3. Is the source, as described in the application, the entire source for 20.2.70, 20.2.72, 20.2.73, or 20.2.74 NMAC applicability purposes?  Yes

- PSD Applicability:

- The source, as determined in 3.0 above, is a minor source before and after this modification.

.

| - History (In descending chronological order, showing NSR and TV): *The asterisk denotes the current active NSR and Title V permits that have not been superseded. |  |  |  |
| --- | --- | --- | --- |
| Permit Number | Issue Date | Action Type | Description of Action (Changes) |
| *9079M1 | 5/16/2024 | Regular-New | The asphalt plant presently operates under General Construction Permit GCP-3-9079. This permit will allow the asphalt plant to operate nighttime hours presently not allowed by the General Construction Permit 3. |
| RE03 9079 | 10/14/2022 | Relocation |  |
| RE02 9079 | 2/23/2022 | Relocation |  |
| RE03 9079 | 10/4/2021 | Relocation |  |
| 9079 | 1/27/2021 | GCP3-New |  |

- Public Response/Concerns: As of March 8, 2024, or the issuance date of this permit, this permit writer has received public comment from 23 citizens.

| - Compliance Testing: |  |  |
| --- | --- | --- |
| Unit No. | Compliance Test | Test Dates |
| 19, 17, and 18 | 20.2.61 Opacity Tests | Following startup |
| 17 and 18 | Initial Compliance Tests | Initial compliance tests shall be conducted within sixty (60) days after the unit(s) achieve the maximum normal production rate. If the maximum normal production rate does not occur within one hundred twenty (120) days of source startup, then the tests must be conducted no later than one hundred eighty (180) days after initial startup of the source. |

- Startup and Shutdown:

  - If applicable, did the applicant indicate that a startup, shutdown, and emergency operational plan was developed in accordance with 20.2.70.300.D(5)(g) NMAC? N/A

  - If applicable, did the applicant indicate that a malfunction, startup, or shutdown operational plan was developed in accordance with 20.2.72.203.A.5 NMAC? Yes

  - Did the applicant indicate that a startup, shutdown, and scheduled maintenance plan was developed and implemented in accordance with 20.2.7.14.A and B NMAC? Yes

  - Does the facility have emissions due to routine or predictable startup, shutdown, and maintenance? No. If so, have all emissions from startup, shutdown, and scheduled maintenance operations been permitted? No, the permittee states:  No SSM emissions are predicted for this permit application.  All control systems will be operational prior to the start or shutdown of asphalt production or aggregate processing.  Maintenance will be performed during period with no production.

- Compliance and Enforcement Status [Title V and NSR/PSD new or modification. Use this email template:

  - This is a new facility.

  - Also, per 2/7/2024 email from Alejandra Avila in the Enforcement Section, “No enforcement action for that Facility in our database.”

- Modeling:   Sahil Kassanjee performed the modeling review.  The following is from his report, dated 5/15/2024:

Modeling Assumptions:

The facility may operate in a year starting from February till the end of November, 10 hours in a day,

anytime of a day. There are no restrictions of operations during daylight or night hours.

Permit Conditions:

The facility may operate during a calendar year starting from February 1st till November 30th, 10

hours in a day, anytime of a day. There are no restrictions of operations in operating during daylight

or night hours. The facility may operate up to 2700 hours per year. This limitation on operating hours

does not apply to use of the hot oil heater (asphalt heater) which may operate 24 hours a day from

February 1st till November 30th. The haul road may be used 24 hours a day but limiting the VMT to

the supporting emission calculations provided in the application.

Upon relocation to other locations, all emission units except the haul road shall be at least 178

meters from the north fenceline, 210 meters from the south fenceline, 155 meters from the west

fenceline, and 137 meters from the east fenceline. Page 11 of the UA4 form provides a diagram for

additional clarification.

Conclusion:

This modeling analysis demonstrates that operation of the facility described in this report neither

causes nor contributes to any exceedances of applicable air quality standards. The standards

relevant at this facility are NAAQS for CO, NO2, O3, Pb, PM10, PM2.5, and SO2; NMAAQS for CO,

H2S, NO2, and SO2; and Class I and Class II PSD increments for NO2, and PM10. 20.2.72.400-499

NMAC establishes permitting requirements for State Toxic Air Pollutants (TAPs) which are identified

in 20.2.72.502 NMAC. The regulations require a source to conduct modeling to predict the

concentration of a TAP if its potential emission rate is greater than the screening level identified in

that section. The screening level may be adjusted by the stack height correction factor listed in

20.2.72.502 NMAC. If a source must model the concentration of a TAP, the TAP is not expected to

pose an environmental concern, and no further action is required, if its concentration remain below

one percent of the Occupational Exposure Limit (OEL) for that TAP. For this application, modeling

demonstrates that the concentrations of Asphalt Fumes remain below one percent of the OEL. See

UA4 for asphalt fume analysis.

Action: The permit can be issued based on this modeling analysis.

- State Regulatory Analysis(NMAC/AQCR):

| Citation 20 NMAC | Title | Applies (Y/N) | Unit(s) or Facility | Justification: |
| --- | --- | --- | --- | --- |
| 2.1 | General Provisions | Yes, Always | Entire Facility | The facility is subject to Title 20 Environmental Protection Chapter 2 Air Quality of the New Mexico Administrative Code so is subject to Part 1 General Provisions, Update to Section 116 of regulation for Significant figures & rounding. Applicable with no permitting requirements. |
| 2.3 | Ambient Air Quality Standards | No for both NSR and TV |  | NSR: 20.2.3 NMAC is a SIP approved regulation that establishes State standards. The NMAAQS themselves are not an “applicable requirement” with which a source must directly comply. The promulgation of a NMAAQS does not, in and of itself automatically result in actionable measures to a source. Instead, the specific measures contained in each individual issued permit, other state promulgated rules, and NM’s EPA approved SIP are the relevant applicable requirements (such as NSPS and MACT). Title V: 20.2.3.9 NMAC, LIMITATION OF APPLICABILITY TO 20.2.70 NMAC. The requirements of NMAAQS are not applicable requirements under 20.2.70 NMAC, as defined by 20.2.3.9 NMAC, 20.2.3.9 NMAC does not limit the applicability of this part to sources required to obtain a permit under the minor NSR regulation, 20.2.72 NMAC, nor does it limit which terms and conditions of NSR permits issued pursuant to 20.2.72 NMAC are applicable requirements in a Title V permit. |
| 2.7 | Excess Emissions | Yes | Entire Facility (Except for Sections 6(b); 110(b)(15); 111; 112; 113; 115; and 116 that are State Enforceable Only) | Applies to all facilities' sources. Federally enforceable except for Sections 6(b); 110(b)(15); 111; 112; 113; 115; and 116 that are State Enforceable Only. |
| 2.11 | Asphalt Process Equipment | Yes | Units 13 (C4), 14 (C5) | The objective of this Part is to establish particulate matter emission standards for asphalt process equipment. These sources are subject to 20.2.11.108 NMAC and 20.2.11.109 NMAC. |
| 2.61 | Smoke and Visible Emissions | Yes | Units 17, 18, 19 | This regulation that limits opacity to 20% applies to Stationary Combustion Equipment, such as engines, boilers, heaters, and flares unless your equipment is subject to another state regulation that limits particulate matter such as 20.2.19 NMAC (see 20.2.61.109 NMAC). |
| 2.70 | Operating Permits | No |  | The source is not a Title V Major Source as defined at 20.2.70.7 NMAC. |
| 2.71 | Operating Permit Fees | No |  | Source is NOT subject to 20.2.70 NMAC as cited at 20.2.71.109 NMAC. |
| 2.72 | Construction Permits | Yes | Entire Facility | NSR Permits are the applicable requirement, including 20.2.72 NMAC. |
| 2.73 | NOI & Emissions Inventory Requirements | Yes | Entire Facility | Applicable to all facilities that require a permit. PER > 10 tpy for a regulated air contaminant. Emissions Inventory Reporting: 20.2.73.300 NMAC applies. |
| 2.75 | Construction Permit Fees | Yes | Entire Facility | This facility is subject to 20.2.72 NMAC |
| 2.77 | New Source Performance Standards | Yes | See Sources subject to 40 CFR 60 | Applies to any stationary source constructing or modifying and which is subject to the requirements of 40 CFR Part 60. |
| 2.78 | Emissions Standards for HAPs | No | See Sources subject to 40 CFR 61 | This regulation applies to all sources emitting hazardous air pollutants, which are subject to the requirements of 40 CFR Part 61. This facility doesn’t emit hazardous air pollutants which are subject to the requirements of 40 CFR Part 61. |
| 2.79 | Permits Nonattainment Areas | No |  | This facility is located in an Attainment Area. |
| 20.2.80 NMAC | Stack Heights | Yes | Units 13 (C4), 14 (C5), 17, 18, 19 | The objective of this Part is to establish requirements for the evaluation of stack heights and other dispersion techniques in permitting decisions. The Department shall give no credit for reductions in emissions due to the length of a source's stack height that exceeds good engineering practice or due to any other dispersion technique. The facility will meet all requirements of good engineering practices. |
| 2.82 | MACT Standards for Source Categories of HAPs | Yes | See sources subject to 40 CFR 63 | This regulation applies to all sources emitting hazardous air pollutants, which are subject to the requirements of 40 CFR Part 63. |

- Federal Regulatory Analysis:

| Federal Regulation | Title | Applies (Y/N) | Unit(s) or Facility | Comments |
| --- | --- | --- | --- | --- |
| Air Programs Subchapter C (40 CFR 50) | National Primary and Secondary Ambient Air Quality Standards | No | N/A | The modeling and conditions developed from the modeling are the applicable requirements to demonstration compliance with the NAAQs. |
| NSPS Subpart A (40 CFR 60) | General Provisions | Yes | See sources subject to a Subpart in 40 CFR 60 | Applies if any other subpart applies. |
| 40 CFR Part 60 Subpart OOO | Standards of Performance for Nonmetallic Mineral Processing Plants | No |  | The provisions of this subpart are applicable to the following affected facilities in fixed or portable nonmetallic mineral processing pants: crushers, grinding mills, screening operations, bucket elevators, belt conveyors, bagging operations, storage bins, enclosed truck or railcar loading stations. (This facility does not have a crusher already present) |
| 40 CFR 60, Subpart I | Performance Standards for Hot Mix Asphalt Facilities | Yes | Units 13 (C4) 14 (C5) | The affected facility, that commences construction or modification after June 11,1973, to which the provisions of this subpart apply is each hot mix asphalt facility. For the purpose of this subpart, a hot mix asphalt facility is comprised only of any combination of the following: dryers; systems for screening, handling, storing, and weighing hot aggregate; systems for loading, transferring, and storing mineral filler, systems for mixing hot mix asphalt; and the loading, transfer, and storage systems associated with emission control systems. |
| 40 CFR Part 60 Subpart IIII (Quad-I) | Standards of Performance for Stationary Compression Ignition Internal Combustion Engines | Yes | Units 17, 18 | (a) The provisions of this subpart are applicable to manufacturers, owners, and operators of stationary compression ignition (CI) internal combustion engines (ICE) as specified in paragraphs (a)(1) through (3) of this section. For the purposes of this subpart, the date that construction commences is the date the engine is ordered by the owner or operator. Link to regulation - read more |
| NESHAP Subpart A (40 CFR 61) | General Provisions | No |  | Applies if any other subpart applies. |
| MACT Subpart A (40 CFR 63) | General Provisions | Yes | See sources subject to a Subpart in 40 CFR 63 | Applies if any other subpart applies. |
| 40 CFR 63 Subpart ZZZZ (Quad Z) | National Emissions Standards for Hazardous Air Pollutants for Stationary Reciprocating Internal Combustion Engines (RICE MACT) | Yes | Units 17, 18 | Facilities are subject to this subpart if they own or operate a stationary RICE, except if the stationary RICE is being tested at a stationary RICE test cell/stand. Applicable if the units are located at the same location for a period of 12 months. |

- Exempt and/or Insignificant Equipment that do not require monitoring:

NSR Exempt Equipment (not entered into Tempo database)

| Unit Number | Source Description | Make | Model No. | Max Capacity | List Specific 20.2.72.202 NMAC Exemption (e.g. 20.2.72.202.B.5) | Date of Manufacture /Reconstruction2 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Serial No. | Capacity Units | Insignificant Activity citation (e.g. IA List Item #1. a) | Date of Installation /Construction2 |
| T3 | Burner Fuel Tank | NA | NA | 20,000 Gallons | 20.2.72.202.B.2.a | Jan-2021 |
|  |  |  | NA | 20,000 Gallons | NA | 2024 |
| T5 | Diesel Fuel Tank | NA | NA | 10,000 Gallons | 20.2.72.202.B.2.a | Jan-2021 |
|  |  |  | NA | 10,000 Gallons | NA | 2024 |
| T6 | Evotherm Storage Tank | NA | NA | 5,000 Gallons | NA | 2024 |
|  |  |  | NA | 5,000 Gallons | 1.a | 2024 |
| T7 | Water Storage Tank | NA | NA | 5,000 Gallons | NA | Jan-2021 |
|  |  |  | NA | 5,000 Gallons | 1.a | 2024 |

- New/Modified/Unique Conditions (Format: Condition#: Explanation):

  - Conditions are based on manager approved conditions from most recently issued HMA permit NSR 9720.

  - Permit Part A Template version: 02/20/2024.

  - Permit Parts B&C Template Version: 12/1/2022.