NMED SOLID WASTE BUREAU

MINIMUM TEST PARAMETERS GUIDANCE FOR WATER TREATMENT PLANT SLUDGE

Revised 3/9/11

- USEPA Paint Filter Liquids test – Method 9095 (no free liquids)

- Percent Solids (no specific limits)

- pH (within range of 2.0 to 12.5)

- TCLP, USEPA Method 1311

  - Arsenic (not to exceed RCRA TCLP level 5.0 mg/L)

  - Cadmium (not to exceed RCRA TCLP level 1.0 mg/L)

  - Chromium (not to exceed RCRA TCLP level 5.0 mg/L)

  - Lead (not to exceed RCRA TCLP level 5.0 mg/L)

  - Mercury (not to exceed RCRA TCLP level .2 mg/L)

5.	Additional sampling parameters may be added by the Department as may be required 	on a case by case review basis.